HISTORY OF THE IGEX-II TWO KILOGRAM Ge DETECTORS RG-I, RG-II, ANDRG-III ENRICHED TO 86% IN 76Ge

RICOGRANDE-I

Crystal grown Sept. 25, 1993 at Oxford Inc. in Oak Ridge, Tenn.

Detector completed November 2nd, 1993

Serial Number 28005-S

Diameter 77.6 mm, Length 84.3 mm

Total Mass 2149.9 g

Dead Layer ~ 800µ

Energy Resolution (FWHM) 2.16 keV at 1332 keV

Operating Voltage (+ 5,000 Volts)

Installed in Homestake Gold Mine at 4000 mwe on Nov. 9, 1993.

Removed from Homestake on 15 June, 1997

Installed in Canfranc 15 July, 1997.

RICOGRANDE- II

Crystal grown Feb. 16th, 1994 at Oxford Inc. in Oak Ridge, Tenn.

Detector Completed May 10th, 1994

Serial Number 28006-S

Diameter 78.6 mm, Length 84.0 mm

Total Mass 2194.0 g

Dead Layer ~800 µ

Energy Resolution (fwhm) 2.37 keV at 1332 keV

Operating Voltage (+4000 Volts)

Installed in Homestake Gold Mine May 22nd, 1994

Removed from Homestake December 27th, 1996

Installed in Canfranc at 2350 mwe January 25th, 1997.

RICOGRANDE-III

Crystal grown Decenber 1st, 1994

Detector completed on or about 15 April, 1994

Serial 28007-S

Diameter 79.2 mm, Length 82.5 mm

Total Mass 2121.0 g

Dead Layer ~ 500 µ

Energy Resolution (fwhm) 2.13 keV at 1332 keV

Operating Voltage (+3800)

Sent directly from Oak Ridge to Canfranc; installed in Canfranc May 10th, 1995